| How It Was Done! |  | Your name Brad Chapman |
| --- | --- | --- |
| Conversion performed by Brad Chapman |  | Town/city and state Bicton Western Australia |
| Year and model of car 1975 MG BGT |  | E-mail address bchapman@y7mail.com |
| Engine (make, model, configuration, displacement, etc.) Rover 3.5 |  | Engine electrical Rebuilt SD1 starter motor. Pre facelift P6 alternator bracket to fit alternator in correct posi, Pertronix pick up in dizzy |
| Intake (manifold & carburetor, or fuel-injection specifics) Holley 600cfm vacuum secondaries |  | Cooling system Modified 4cly (4 core) relocated outlets |
| Cam Unknown but feels like Stage 1 or 2 |  | Exhaust system Block hugger extractors, headers and twin system |
| Transmission (type, make/model, ratio of top gear, etc.) Toyota Supra Alloy case with Dellow Bellhousing |  | Driveshaft Larger yokes welded and balanced to std tail shaft |
| Clutch System Dellow Clutch kit inc fork, bearing carrier etc |  | Rear axle (make/model, ratio, limited-slip?) Std but plans to upgrade to LSD |
| Front suspension Std |  | Brakes V8 front calipers |
| Rear suspension Std |  | Wheels/Tires Mini lite 15 x 6 |
| Weight 1020kg (2248Lbs) |  | Electrical (gauges, lighting, audio, etc.) |
| Miles driven since completion |  | Date conversion completed February 2009 |
| Other chassis modifications Relocated radiator mounts and cut and welded chassis rails to de-bumper vehicle |  |  |
| Body and interior modifications Sebring front and rear valances. Removed the heavy rubber bumpers. |  |  |
| Performance data |  |  |
| Sources of parts / information British v8 of course, parts sourced through many Australian suppliers and websites |  |  |
| Comments / tips / notable problems encountered during or after completion… Where to start! I think the main area of concern I had was the engine location and gearbox selection. I decided to use a Supra gearbox which are readily available in Australia and come in 4 different length casings ( 18,19,20,21 inch). The longest is the most appropriate and located the gear stick very near the original location. The box I used is referred to as a w55 box and has very similar gear ratios to the RV8 model. The boxes are good for about 500hp so are virtually break proof. At the time of writing I have not upgraded the differential but intend to do this soon. I purchased the alloy V8 engine mounts from MGOC website. I think in hind sight I could have placed the motor slightly further back , maybe an inch or so. However the height is good and I have enough clearance to remove the sump pan with out lifting the engine. The Dellow bell housing I used was a easy install and clears the tunnel by about 10-12mm either side so I did not have to modify the tunnel at all. Dellow also supplied the clutch, fork, slave cylinder and rear gearbox cross member. |  |  |

Note: Please send us as many photos as you can!  Photo captions are great too.                                          www.britishv8.org