| How It Was Done! |  | Your name Bryant Ercanbrack |
| --- | --- | --- |
| Conversion performed by Purchased assembled but in rough shape |  | Town/city and state Eagle Mountain, UT |
| Year and model of car 1964 MG B |  | E-mail address bryantercanbrack@hotmail.com |
| Engine (make, model, configuration, displacement, etc.) Ford 289 V8 |  | Engine electrical Painless wiring “T” Bucket harness, Mallory ignition, cap and rotor, |
| Intake (manifold & carburetor, or fuel-injection specifics) 4 bbl Edlebrock intake and carberator |  | Cooling system Large radiato/r acc. fan |
| Cam |  | Exhaust system Custom headers to pipes under doors |
| Transmission (type, make/model, ratio of top gear, etc.) 4 speed |  | Driveshaft |
| Clutch System Stock pedal, clutch flywheel and ring gear from 289 |  | Rear axle (make/model, ratio, limited-slip?) Unsure make or model 2:1 ratio |
| Front suspension Stock |  | Brakes stock |
| Rear suspension Stock |  | Wheels/Tires |
| Weight |  | Electrical (gauges, lighting, audio, etc.) New Equuis gauges, original speedometer, changed to 12V negative ground, Painless wiring “T” bucket harness |
| Miles driven since completion 50 miles this time |  | Date conversion completed Purchased in rough shape June 08 finished in Sept. 08 |
| Other chassis modifications |  |  |
| Body and interior modifications Radio installed, speakers in rear, Chromed roll bar and exhaust covers, all new interior, carpet, seats, side panels |  |  |
| Performance data Original motor (289 V8) 200hp when new |  |  |
| Sources of parts / information Moss motors |  |  |
| Comments / tips / notable problems encountered during or after completion… Headers are a pain to remove and replace because of room allowed (none!) Engine and tranny are removed as one. After purchased removed engine and tranny to replace stripped ring gear/starter, painted, rewired, redid interior, new gauges, hood, latches, battery behind drivers seat, new chrome grill, emblems, etc. |  |  |

Note: Please send us as many photos as you can!  Photo captions are great too.                                          www.britishv8.org