| How It Was Done! |  | Your name Submitted by: Dave Livinghouse |
| --- | --- | --- |
| Conversion performed by Butch Pilgrim, P&M Auto, LaPorte, Colo |  | Town/city and state Ft. Collins, Colo |
| Year and model of car 1967 MG-B |  | E-mail address ddlivinghouse@wmconnect.com |
| Engine (make, model, configuration, displacement, etc.) Buick 231 V-6 140 HP |  | Engine electrical H.E.I ignition with all new wiring |
| Intake (manifold & carburetor, or fuel-injection specifics) Edlebrock intake and Edlebrock 650 4 bbl carb |  | Cooling system Stock radiator lengthened 3 inches with a new core |
| Cam Stock |  | Exhaust system Custom built exhaust manifolds with a rear dump leading into side pipes |
| Transmission (type, make/model, ratio of top gear, etc.) GM 5 speed Manual |  | Driveshaft Stock MG with custom fitted GM front yoke |
| Clutch System Stock |  | Rear axle (make/model, ratio, limited-slip?) Stock |
| Front suspension Stock |  | Brakes Stock |
| Rear suspension Stock |  | Wheels/Tires Wire Rims with stock size tires |
| Weight Stock weight plus a few extra pounds for the engine and tranny |  | Electrical (gauges, lighting, audio, etc.) All new wiring from engine compartment to custom dash and gauges with AM-FM CD Player |
| Miles driven since completion About 500 |  | Date conversion completed 2005 |
| Other chassis modifications See Below |  |  |
| Body and interior modifications Custom built Roll Bar , and Custom made Black Walnut dash |  |  |
| Performance data |  |  |
| Sources of parts / information All new parts were purchased from Moss Ind. of Calf. |  |  |
| Comments / tips / notable problems encountered during or after completion… This is the Story as was told to me. This Conversion took 2 years from start to finish. Starting with the motor. The only way to mount the V-6 was to do minor alterations to the firewall in order to have the heads clear. The motor had to mounted so the crank pully sits just in front of the MG rack and pinion steering. Once that was accomplished, frame mounts had to be built to accept the GM motor mounts. All of the clearances were minimal. The passengers side of the frame had to be trimmed and boxed to clear the starter and oil filter. The radiator was lengthened 3 inches by way of a new core and moved forward 6 inches and lowered 2 inches to accommodate closing the hood and cooling the engine. A “pusher” electric fan was mounted in front of the radiator, with no problems. The upper radiator hose was spliced to clear the H.E.I. ignition and distributor. There was absolutely no clearence for stock exhaust manifolds, so the manifolds were hand made with a rear dump. The exhaust pipes off the manifold was run through the inner fender well panels then run to the outside of the body and along the rockers. Harley mufflers were then added to the end of the pipes, for sound and looks. A special clutch bracket was made for the GM “Cable” clutch set-up. A GM drive shaft was to be used but would not fit the tunnel because of the MG emergency brake mounting so the front GM yoke had to be machined to fit the MG drive shaft. Now the Hot Rod Body. The front and rear bumpers were removed and replaced with Sebring farings from Moss Industries, which were a great fit with little cosmetic work. A tube grill was hand made for looks. The aluminum hood was replaced with a steel one, from Moss, and a steel scoop was welded to the hood to accommodate the Edlebrock intake and carb. New wiring and instruments were installed in the custom made Black Walnut dash. The body was then painted with PPG Acrylic Urathane Porsche Red with a Black Racing Stripe and then shot with 2 coats of PPG Clear . This theme of the paint was then carried through the upholstery. The upholstery was done by Keith Hall of University Upholstery located in Eaton Colo. It may not sound like a lot of work, but if you have ever done a conversion, you know it takes a lot of time. The Owner of the car has only driven it about 450 miles. The car has about 500 miles total. This Car is up “ FOR SALE” , asking price is $18,000.00. This is the story, and I am sticking to it !!. By the way, I have ridden in this car with Butch driving and it is like a little rocket sled on wheels. Dave Livinghouse Ft. Collins Colo. |  |  |

Note: Please send us as many photos as you can!  Photo captions are great too.                                          www.britishv8.org