| How It Was Done! |  | Your name Ken Ritacco |
| --- | --- | --- |
| Conversion performed by Ken Ritacco |  | Town/city and state Westborough, Ma |
| Year and model of car 1977 MGB |  | E-mail address ritacco0758@charter.net |
| Engine (make, model, configuration, displacement, etc.) Buick 215 V8, 10.5 to 1 compression, Balanced & Blueprinted |  | Engine electrical Relays added to headlights, horn and fans |
| Intake (manifold & carburetor, or fuel-injection specifics) Buick 4V intake |  | Cooling system V8 radiator |
| Cam |  | Exhaust system Coated Block hugger headers, magnaflow mufflers, Anza Duel exhaust tips |
| Transmission (type, make/model, ratio of top gear, etc.) WC T5 .63 top gear |  | Driveshaft Custom cut and balanced |
| Clutch System Weber clutch w Hydraulic throw out bearing |  | Rear axle (make/model, ratio, limited-slip?) 8.8” ford Posi traction rear axle |
| Front suspension 2” lowered 550 lb springs, stock shocks with up rated valves Poly bushings |  | Brakes Rebuilt ford rear drum, MGB front, new master |
| Rear suspension 1 ½ lowering blocks, Koni tube shock conversion |  | Wheels/Tires 205 tires American Racing wheels |
| Weight Aprox. 2200 lb |  | Electrical (gauges, lighting, audio, etc.) Stock MGB, reworked tach |
| Miles driven since completion 3000 |  | Date conversion completed 7/23/08 |
| Other chassis modifications Frame connectors added |  |  |
| Body and interior modifications Prestige leather interior Black with Red Piping, Black Wool carpets with red piping |  |  |
| Performance data Aprox. 200 hp |  |  |
| Sources of parts / information Parts from D&D and others. |  |  |
| Comments / tips / notable problems encountered during or after completion… Car started as a rust free example from the west coast. Totally disassembled brought to bar metal sealed primed and painted. Toyota Impulse Red, 6 coats of base and 6 coats of clear. Chrome Bumper conversion. Engine was rebuild, balanced and blueprinted, new internals, and 10.5 to 1 compression. Transmission was inspected and resealed, as was the rear end. All systems were gone through and all parts replaced with new as necessary. Prestige interior was used leather seats upper door panels, wool rugs. Gas tank was moved 2” to the left so duel exhaust could be run. Frame connectors from Fast Cars Inc. were added to stiffen the chassis. Cars runs and handles great and is a blast to drive! |  |  |

Note: Please send us as many photos as you can!  Photo captions are great too.                                          www.britishv8.org